/** \page i_simulink Notes on Simulink/Real-Time Workshop generated code

\section i_simulink_purpose Purpose of this document

Simulink models, converted with real-time workshop, are a quick means

of creation of c-code for real-time simulation. This document

describes some of the work and considerations for use of these codes

in a DUECA/DUSIME environment.

\section i_simulink_multitasking Multitasking RTW

Dueca has its own mechanisms for splitting work over several threads,

and crossing this with the Real-Time Workshop mechanisms is not a good

idea. If it happens, the following code should be a starting point:

\code

// determine which is the first task id

# if TID01EQ == 1

#  define FIRST_TID 1

# else

#  define FIRST_TID 0

# endif

// calculate time of next time step, and pass it to the solver

real_T tnext = rt_UpdateDiscreteEvents(S);

ssSetSolverStopTime(S, tnext);

// calculate outputs and do update for base sample time

sfcnOutputs(S, FIRST_TID);

sfcnUpdate(S, FIRST_TID);

// do either a numerical integration, or discrete time update

if (ssGetSampleTime(S, 0) == CONTINUOUS_SAMPLE_TIME) {

rt_UpdateContinuousStates(S);

}

else {

rt_UpdateDiscreteTaskTime(S, 0);

}

#if FIRST_TID == 1

rt_UpdateDiscreteTaskTime(S, 1);

#endif

// update for the other sample times

for (int it = FIRST_TID+1; it < NUMST; it++) {

if (ssGetSampleHitPtr(S)[it]) {

sfcnOutputs(S,it);

sfcnUpdate(S,it);

rt_UpdateDiscreteTaskTime(S, it);

}

}

\endcode

For the RTW 4 version, the following:

\code

// determine which is the first task id

# if TID01EQ == 1

#  define FIRST_TID 1

# else

#  define FIRST_TID 0

# endif

// calculate time of next time step, and pass it to the solver

real_T tnext = rt_SimUpdateDiscreteEvents(rtmGetNumSampleTimes(S),

rtmGetTimingData(S),

rtmGetSampleHitPtr(S),

rtmGetPerTaskSampleHitsPtr(S));

rtsiSetSolverStopTime(rtmGetRTWSolverInfo(S), tnext);

// calculate outputs and do update for base sample time

rtmiOutputs(rtmGetRTWRTModelMethodsInfo(S), FIRST_TID);

rtmiUpdate(rtmGetRTWRTModelMethodsInfo(S), FIRST_TID);

// do either a numerical integration, or discrete time update

if (rtmGetSampleTime(S,0) == CONTINUOUS_SAMPLE_TIME) {

rt_ODEUpdateContinuousStates(rtmGetRTWSolverInfo(S));

}

else {

rt_SimUpdateDiscreteTaskTime(rtmGetTPtr(S),

rtmGetTimingData(S), 0);

}

#if FIRST_TID == 1

// do the discrete time update for the no 1 task

rt_SimUpdateDiscreteTaskTime(rtmGetTPtr(S),

rtmGetTimingData(S), 1);

#endif

// do the updates for other sample times

for (i=FIRST_TID+1; i<NUMST; i++) {

if (rtmGetSampleHitPtr(S)[it]) {

rtmiOutputs(rtmGetRTWRTModelMethodsInfo(S), i);

rtmiUpdate(rtmGetRTWRTModelMethodsInfo(S), i);

rt_SimUpdateDiscreteTaskTime(rtmGetTPtr(S),

rtmGetTimingData(S), i);

}

}

#endif

\endcode

*/